GLOUCESTER TOWNSHIP FIRE DISTRICT #2

BUREAU OF FIRE PREVENTION

43 Somerdale Road, Blackwood, NJ 08012

Phone 856-282-3370 Ext. 124 Fax 856-545-7485 Email gtfm82@gtfd2.com

Smoke Detector, Carbon Monoxide Detector & Secondary Power Source Identification

Property Address:_______________________________________________________________________________________________________

Owner Name:___________________________________________________________Phone #_______________________________________

Tenant   yes/no   Secondary Power Source  yes/no   Type:___________________________________________________________

Settlement/Rental Date:___________________________ Occupied (   )   Vacant  (   )  Lock Box Code:_____________________

Contact for appointment:_______________________________________________________________________________________________

Contact phone #__________________________________Email:_____________________________________________@________________

Inspection Checklist - House numbers are numerical characters, contrasting color from the house, at least 4” high, permanently affixed to the house and visible from the street. Light post or mailbox numbers are NOT acceptable. Smoke Alarms - Battery Operated smoke detectors shall be of the ten (10) year sealed battery type. - Smoke alarm located on every level of the dwelling - Basement level alarm is located at bottom of stairwell, at ceiling level (not between joists) - Bedroom hallway alarms MUST be within 10 feet of ALL bedroom doors - Electric smoke alarms (hardwired) CANNOT be replaced with battery operated alarms - Do NOT install smoke alarms in kitchens, bathrooms, near forced air ducts or furnaces or in the “dead air” space where the ceiling meets the wall, close to ceiling fans, in crawl spaces or attics. Carbon Monoxide Alarms - Can be combination detector- smoke/CO - Located in the hallway, within 10 feet of ALL bedroom doors - Installed per manufacturer’s instructions - Carbon monoxide alarms are less than 7 years old - Do NOT place alarms in electrical outlets that can be turned off by a switch or that are located against floor molding. Plug-in battery powered and hardwired CO alarms are acceptable. Official Use Only_______________ Date Received:___________________________ Cash_________or Check #_____________ Receipt #_____________By:____________ Scheduled:______________at:__________ Secondary Power Source Identification - A secondary power source label must be installed within 18” of the main electrical panel and another within 18” of the electrical meter - Secondary Power sources may include permanently installed internal combustion generators, solar panels, battery storage systems or any other supplemental source of electrical energy to the primary power supply - Labels must be marked “CAUTION: MULTIPLE SOURCES OF POWER” Labels may not be handwritten Fees *** Based on Business Days not including settlement day*** Applications received 10 or more business days prior to settlement $35.00 Applications received 5-9 business days prior to settlement $70.00 Applications received 4 or less business days prior to settlement $125.00 *Applications received under 4 days will be completed as manpower permits.

Please include exact cash or check made payable to GTFD#2

ESP # ___________________ updated  4/16/2025