SMOKE AND CARBON MONOXIDE

DETECTOR PLACEMENT GUIDE

Where to locate detectors:

Detectors are to be located on every level of a residence and outside each bedroom.  If all the bedrooms are located down the same hallway the detector may be located in the common hallway but must be within 10 feet from all bedroom doors.  If the residence has two separate sleeping areas, a detector is required to be located outside each sleeping area.  A carbon monoxide detector should also be installed outside of each sleeping area.   Carbon Monoxide detectors must be less than 7 years old and operational.

Note:  If the property was built and a Certificate of Occupancy was issued with hard wired smoke detectors, the residence must maintain that same level of fire alarm protection.  You may not replace hardwired detectors with battery detectors.  If the detector is older than 10 years of age it shall be replaced.  Please refer to information on the back for any other placement questions or contact your local fire marshal.  In addition, effective January 1, 2019 all battery operated smoke detectors must be the 10 year sealed battery type.

Where NOT to locate detectors:

To avoid false alarms and/or improper operations, avoid installation of smoke detectors in the following areas:

Kitchen-smoke from cooking may cause a nuisance alarm.

Bathrooms-excessive steam from a shower may cause a nuisance alarm.

Around HVAC ducts-detectors shall not be located within 3 feet as air movement may prevent smoke from reaching the detector.

Near furnaces of any type-air/dust movement and normal combustion products may cause a nuisance alarm.

The “dead air” space where the ceiling meets the wall (4” from corner).

The Peak of an “A” frame type ceiling.  “Dead Air” may prevent smoke from reaching the detector.

Secondary Power Source Identification:

A Secondary Power Source may include permanently installed internal combustion generators, solar panels, battery storage systems, or any other supplemental sources of electrical energy to the primary power supply.  A label must be installed within 18 inches of the main electrical panel and an additional label within 18” of the electrical meter.  Labels may not be handwritten and must be marked with the wording similar to “CAUTION: MULTIPLE SOURCES OF POWER”

Additional information may be found on our Fire District Website at GTFD2.org under the Fire Prevention Bureau Section